AI Homework Supervision Bot

Full Technical Build Plan

Architecture • Models • APIs • Hardware • Weaknesses • Roadmap

June 2026 | Nanjing Company Project

# 1. What This Product Is

The AI Homework Supervision Bot is a real-time computer-vision system that monitors a child studying at their desk. It runs the Pomodoro Technique (timed focus and break cycles), detects distraction behaviors via camera, generates intelligent voice reminders, and reports session data to parents. It is NOT a recording device no video is ever stored.

Two product forms:

- Hardware Desk Bot: A physical device placed on the child's desk with a camera, speaker, microphone, and small expression screen. The child interacts with it like a companion robot.

- Lightweight App: A phone or tablet app using the built-in camera. Same detection logic, no dedicated hardware. Lower cost, faster to ship.

The pipeline in one line: Camera Feed → Computer Vision → Feature Extraction → Rules Engine → Script Generation → TTS Voice → Expression Screen → Parent Dashboard

# 2. Hardware Specifications (Desk Bot Form)

The app form skips this section entirely and runs on any Android/iOS device with a front-facing camera.

## 2.1 Compute Board

Two viable options depending on budget and performance requirement:

| Component | Option A (Recommended) | Option B (Budget) | Notes |
| --- | --- | --- | --- |
| Compute Board | Raspberry Pi 5 (4GB or 8GB) | Raspberry Pi 4B (4GB) | Pi 5 is ~3x faster than Pi 4 for ML inference |
| Inference Accelerator | Hailo-8L M.2 HAT (13 TOPS) | None (CPU only) | Hailo HAT enables 30fps real-time YOLOv8 inference on Pi 5 |
| Camera | Raspberry Pi Camera Module 3 (12MP, autofocus) | USB 1080p 30fps webcam | CSI camera has lower latency; USB is plug-and-play |
| Display | 4-inch IPS TFT LCD, 480x320 (SPI or DSI) | 3.5-inch TFT, 320x480 | Shows character expressions; IPS gives better viewing angle |
| Microphone | ReSpeaker 2-Mic HAT or USB mic array | Single USB microphone | Array gives better voice wake detection with noise cancellation |
| Speaker | 3W speaker + MAX98357A I2S amp HAT | USB speaker | I2S gives cleaner audio than USB on Pi |
| Storage | 32GB microSD (A2 class) | 16GB microSD | A2 class for faster random read/write |
| Power | 5V/5A USB-C power adapter | 5V/3A USB-C | Pi 5 needs 5A; undervolting causes instability |
| Enclosure | Custom 3D-printed housing | Off-shelf project box + mods | Design should angle camera at child's face from desk level |

## 2.2 Cost Estimate

| Item | Cost (USD) | Cost (CNY approx.) |
| --- | --- | --- |
| Raspberry Pi 5 (4GB) | $60 | ~430 |
| Hailo-8L HAT (optional) | $50 | ~360 |
| Camera Module 3 | $25 | ~180 |
| 4" TFT Display | $18 | ~130 |
| Microphone HAT | $15 | ~108 |
| Speaker + amp | $8 | ~58 |
| microSD + power supply | $15 | ~108 |
| 3D print + enclosure | $20 | ~144 |
| TOTAL (with Hailo) | ~$211 | ~1,518 |
| TOTAL (without Hailo) | ~$161 | ~1,158 |

# 3. Full Software Stack

Everything below runs on the device. No cloud dependency for the core detection loop. Cloud is optional for the parent dashboard sync and LLM script generation.

| Layer | Technology | License | Cost | Purpose |
| --- | --- | --- | --- | --- |
| OS | Raspberry Pi OS Lite (64-bit) | Open source | Free | Base operating system |
| Language | Python 3.11 | PSF | Free | Main application language |
| Camera capture | OpenCV 4.9 (cv2) | Apache 2.0 | Free | Frame grab, resize, preprocessing |
| Face landmarks | Media Pipe Face Mesh | Apache 2.0 | Free | 468 face landmarks per frame |
| Body pose | Media Pipe Pose (Blaze Pose) | Apache 2.0 | Free | 33 body landmarks per frame |
| Object detection | YOLOv8n (Ultralytics) | AGPL-3.0 | Free | Detects: person, cell phone, book |
| Optical flow | OpenCV (Farneback / LK) | Apache 2.0 | Free | Writing activity score |
| State machine | Python transitions library | MIT | Free | Pomodoro flow management |
| Rules engine | Custom Python class | Proprietary | Free | Event thresholds and triggers |
| Script generation (MVP) | Template strings + Python random | Proprietary | Free | Pre-written reminder scripts |
| Script generation (v2) | Claude API (claude-haiku-4-5) | Anthropic TOS | ~$0.001/call | LLM-generated personalized scripts |
| TTS (primary) | edge-tts (Microsoft) | MIT | Free* | High-quality Chinese voices |
| TTS (offline fallback) | pyttsx3 | MIT | Free | Offline TTS, lower quality |
| Audio playback | pygame.mixer or playsound | LGPL | Free | Play TTS audio output |
| Expression display | pygame + PIL/Pillow | LGPL/PIL | Free | Render animations on TFT |
| Data storage | SQLite (via Python sqlite3) | Public domain | Free | Local session and event logs |
| Parent dashboard backend | FastAPI (Python) | MIT | Free | REST API for dashboard |
| Parent dashboard frontend | React + TailwindCSS | MIT | Free | Web-based parent view |
| Mobile parent app | WeChat Mini Program | Tencent TOS | Free | Recommended for CN market |
| Cloud sync (optional) | Supabase (Postgres + Auth) | Apache 2.0 | Free tier | Remote parent access |

* edge-tts requires internet. For fully offline operation, use pyttsx3 or bundle a local TTS model like Coqui TTS.

# 4. Computer Vision Pipeline Model-by-Model

This is the core engineering challenge. Every behavioral signal the bot detects is derived from one of the perception modules below. No single model does everything signals are combined in the Feature Aggregation layer.

## 4.1 Media Pipe Face Mesh

Library: google-mediapipe. Installed via: pip install mediapipe

What it gives you: 468 3D face landmarks per frame at 30fps on CPU (faster with GPU). No training required. Works in real time on Raspberry Pi 5.

What you derive from it:

- Head Pose (Pitch/Yaw/Roll): Use solvePnP (OpenCV) with 6 landmark reference points (nose tip, chin, eye corners, mouth corners) and a known 3D face model. Output: rotation vector. Pitch > 20deg = looking down/up. Yaw > 30deg = looking sideways.

- Gaze Direction: Track iris landmark positions (landmarks 468-477 in full mesh) relative to eye corners. If iris is consistently at edge of eye = looking away.

- Eye Closure Ratio (EAR): Distance between upper and lower eyelid landmarks divided by eye width. EAR < 0.2 sustained for 2+ seconds = drowsy/eyes closed.

- Face Presence: If face detection confidence drops below 0.5 = child left seat or turned away from camera.

- Blink rate: Count blinks per minute. Very low blink rate = screen-staring / fatigue risk.

## 4.2 MediaPipe Pose (BlazePose)

Library: same mediapipe package. Model: BlazePose Full (33 landmarks). Slightly heavier than Face Mesh but still runs in real time on Pi 5.

What you derive from it:

- Seat Presence: Are shoulder landmarks (11, 12) visible in frame and within the ROI? If not = out of seat.

- Slumping posture: Head landmark (0) Y-coordinate significantly below shoulder midpoint Y-coordinate = slouching forward.

- Head-too-close: Z-coordinate of nose tip landmark (converted to approximate distance) closer than threshold = too close to screen.

- Crossed legs: Hip and knee landmark geometry. Detect if knee is significantly higher than expected for seated position.

- Body motion during break: Velocity of limb landmarks across frames. Low motion during entire break = sitting still, need to get up and move.

## 4.3 YOLOv8n (Object Detection)

Library: pip install ultralytics. Model: yolov8n.pt (6MB, pretrained on COCO 80 classes). No custom training needed for Phase 1.

Relevant COCO classes this model already detects:

- Class 67: cell phone primary distraction signal. If phone detected in frame confidence > 0.6 = phone use.

- Class 0: person confirms child is present / for multi-person detection.

- Class 73: book confirms study materials are on desk.

- Class 63: laptop relevant if child studies on laptop (different behavior profile).

Performance on Pi 5 (no Hailo): ~8-12 fps for 640x480 input. Acceptable for event detection (phone use doesn't need 30fps detection). With Hailo-8L: 30+ fps at full 640x640 resolution.

Custom fine-tuning (Phase 2 only, not needed for MVP): If you want to detect specific objects like a specific type of toy or game, collect 200-300 labeled images and fine-tune with: yolo train model=yolov8n.pt data=custom.yaml epochs=50

## 4.4 Optical Flow (Writing Activity Score)

Library: OpenCV (built-in, no extra install). Algorithm: Farneback dense optical flow or Lucas-Kanade sparse optical flow on hand ROI.

How it works:

- In PREPARE phase, the system identifies the desk ROI (the area of the frame containing the desk surface where the child writes).

- During FOCUS, compute optical flow between consecutive frames within the hand/desk ROI.

- Sum the magnitude of flow vectors. High magnitude + localized to hand area = active writing. Low magnitude for 30+ seconds = pen is idle.

No model training required. This is a deterministic algorithm, not ML. The threshold for 'writing vs idle' needs calibration per child (done in the first session via a 2-minute baseline recording).

## 4.5 ROI Setup (PREPARE Phase)

Before the first focus session, the bot guides the child through a 2-minute calibration:

- Child sits at desk in normal study position.

- Bot captures a reference frame and auto-detects: face position, desk surface (homography from ArUco markers or just bounding box), approximate hand writing zone.

- System confirms: lighting adequate (mean brightness > 60), face detected (confidence > 0.8), desk ROI identified.

- ROI coordinates saved to config file. Used for all subsequent sessions.

## 4.6 Feature Aggregation Layer

This is a Python class that takes all raw signals from the above modules and produces a clean feature vector every second. The rules engine reads from this vector, not from raw model outputs.

| Feature | Source | Type | Description |
| --- | --- | --- | --- |
| seat_presence | MediaPipe Pose | Boolean | Are shoulder landmarks in ROI? |
| face_presence | MediaPipe Face Mesh | Boolean | Is face detected with confidence > 0.5? |
| head_pitch | solvePnP (Face Mesh) | Float (degrees) | Forward/back tilt of head |
| head_yaw | solvePnP (Face Mesh) | Float (degrees) | Left/right rotation of head |
| gaze_direction | Iris landmarks | Enum (CENTER/LEFT/RIGHT/DOWN/UP) | Where child is looking |
| eye_closure_ratio | Eye landmarks | Float (0-1) | 0 = fully closed, 1 = fully open |
| phone_detected | YOLOv8n | Boolean + confidence | Phone visible in frame |
| writing_activity_score | Optical flow | Float (0-100) | Rolling 5s average flow magnitude |
| body_motion_score | MediaPipe Pose velocity | Float (0-100) | Movement during break |
| distance_to_screen | Nose Z-coordinate | Float (estimated cm) | Approximate face-to-screen distance |
| posture_slouch | Head-shoulder Y delta | Boolean | Head significantly below shoulder line |

# 5. Pomodoro State Machine — Implementation

Use the Python transitions library (pip install transitions). This gives you a clean declarative state machine with callbacks, guards, and logging built in.

## 5.1 States and Transitions

| State | Entry Action | Active Monitoring | Exit Triggers | Default Duration |
| --- | --- | --- | --- | --- |
| IDLE | Show standby expression, wait for tap or voice wake | None | User starts session | Indefinite |
| PREPARE | Guide child: sit properly, place stationery, run ROI calibration | Lighting check, face detection check | Child confirmed ready OR 2 min timeout | 1-2 minutes |
| FOCUS | Start timer, begin full CV pipeline, enable rules engine | All signals active (phone, gaze, writing, posture, fatigue) | Timer expires OR 3 consecutive serious interruptions | 25 min (configurable) |
| BREAK | Celebrate completion, TTS break script, disable intervention rules | Body motion only (are they actually resting?) | Break timer expires | 5 min (configurable) |
| RESUME_CHECK | Check if child returned to desk | Seat presence + face presence | Child detected at desk OR 3 min timeout | Up to 3 min |
| COMPLETE | Session summary script, send digest to parent dashboard | None | User closes or starts new session | Indefinite |

## 5.2 Configurable Parameters

| Parameter | Default | Range | Notes |
| --- | --- | --- | --- |
| focus_duration_minutes | 25 | 10-45 | Standard Pomodoro is 25 min; younger children may need 15-20 |
| short_break_minutes | 5 | 3-10 |  |
| long_break_minutes | 15 | 10-30 | After every 4 rounds |
| rounds_before_long_break | 4 | 2-6 |  |
| distraction_alert_delay_seconds | 30 | 15-60 | How long a distraction must sustain before alert fires |
| phone_alert_delay_seconds | 5 | 3-15 | Phone detected = alert faster than gaze distraction |
| out_of_seat_alert_delay_seconds | 60 | 30-120 | Brief leave is OK; sustained absence triggers alert |
| fatigue_ear_threshold | 0.2 | 0.15-0.25 | EAR below this = eyes closing |
| fatigue_sustained_seconds | 10 | 5-20 | How long EAR must be low to trigger fatigue alert |
| posture_distance_minimum_cm | 30 | 25-40 | Too close to screen threshold |
| writing_idle_threshold_seconds | 60 | 30-120 | Pen idle this long during focus = distraction suspected |

# 6. Rules Engine How Decisions Are Made

The rules engine reads the Feature Aggregation vector every second and applies threshold-based rules to generate events. Events trigger actions via the script generation module. The engine has three priority levels: LOW (expression change only), MEDIUM (voice reminder), HIGH (firm voice + parent notification).

## 6.1 Focus Phase Rules

| Rule Name | Condition | Priority | Action |
| --- | --- | --- | --- |
| PHONE_DETECTED | phone_detected=True, confidence>0.7, sustained 5s | HIGH | Immediate TTS + log event |
| DISTRACTED_GAZE | gaze! = CENTER for 30s AND face_presence=True | MEDIUM | Gentle TTS reminder |
| OUT_OF_SEAT | seat_presence=False for 60s | HIGH | TTS + log, escalate if 90s |
| WRITING_IDLE | writing_activity_score < 10 for 60s AND seat=True AND face=True | LOW->MEDIUM | Expression first, then voice if 90s |
| FATIGUE_DROWSY | eye_closure_ratio < 0.2 sustained 10s | MEDIUM | TTS encouragement + suggest break early |
| FATIGUE_OVERFOCUS | focus_elapsed > 35 min (if session extended) | LOW | Suggest wrapping up |
| BAD_POSTURE_CLOSE | distance_to_screen_cm < 30 sustained 20s | LOW | Gentle posture reminder |
| BAD_POSTURE_SLOUCH | posture_slouch=True sustained 30s | LOW | Posture reminder TTS |
| GOOD_FOCUS | No events for 10 consecutive minutes | POSITIVE | Encouragement TTS + happy expression |

## 6.2 Break Phase Rules

| Rule Name | Condition | Priority | Action |
| --- | --- | --- | --- |
| BREAK_STUDYING | writing_activity_score > 20 during break | MEDIUM | Tell child to actually rest |
| BREAK_SEDENTARY | body_motion_score < 5 for entire break so far AND break > 3 min | LOW | Encourage getting up and moving |
| BREAK_TOO_CLOSE | distance_to_screen_cm < 25 sustained 30s | LOW | Screen distance reminder |
| BREAK_ENDING | break_elapsed within 30s of end | INFO | Countdown warning TTS |
| BREAK_PHONE | phone_detected=True sustained 30s during break | INFO | Log only (phone OK during break, just track) |

## 6.3 Alert Escalation

To avoid over-alerting (the biggest risk for child rejection), alerts escalate through three stages with increasing firmness and increasing wait time before the next alert on the same topic:

- Stage 1 (first occurrence): Subtle expression change on screen. No voice. Log event.

- Stage 2 (second occurrence within session, or sustained past delay threshold): Gentle, friendly voice reminder. Encouraging tone.

- Stage 3 (three+ occurrences or serious violation): Firm voice reminder. Parent notification sent.

Same event type cannot trigger voice again within 3 minutes unless the behavior stopped and restarted. This cooldown is critical for usability.

# 7. Script Generation Voice Content

This is what the child actually hears. Getting the tone right matters more than accuracy. A child who finds the reminders annoying or condescending will find a way to disable the bot.

## 7.1 Phase 1: Template System (MVP No API Cost)

A Python dictionary maps (event_type, priority_level) to a list of script variants. Each call randomly picks one variant. Child's name and context data are interpolated.

Example template bank (translate to Chinese for production):

- PHONE_DETECTED, HIGH: ['{name}, put down the phone and get back to it!', '{name}, phone away you were doing so well!', 'Hey {name}, the phone can wait. Your homework cannot.']

- DISTRACTED_GAZE, MEDIUM: ['{name}, eyes back on the book', 'Stay with it, {name} — almost there', '{name}, you drifted off. Come back.']

- GOOD_FOCUS, POSITIVE: ['Amazing focus, {name}! That was a great {minutes}-minute stretch.', 'You stayed with it for {minutes} minutes! That is real discipline, {name}.']

Minimum 5 variants per event type to avoid repetition. Total scripts needed: ~15 event types x 3 priority levels x 5 variants = 225 scripts. One-time effort.

## 7.2 Phase 2: LLM Script Generation (Claude API)

Replace the template lookup with a call to the Claude API. Input: structured JSON with event type, child profile (name, age, study preferences), session context (how long they've been going, how many distractions today, current mood/state). Output: a short, natural voice reminder.

Recommended model: claude-haiku-4-5 (fastest response, lowest cost at ~$0.0008 per 1K input tokens). A typical script generation call uses ~200 tokens input + 50 tokens output = cost under $0.001 per reminder.

System prompt structure:

- Role: You are {character_name}, a friendly study companion for {name}, age {age}.

- Tone: Encouraging, direct, age-appropriate. Never scolding. Maximum 2 sentences.

- Context: {event_type} happened. {session_context}.

- Output: A single spoken reminder. No emojis. Output in Chinese (Mandarin).

Fallback: If API call fails or takes > 2 seconds, fall back to template system. Never block the alert on an API response.

Offline option: Run Qwen2-0.5B or Llama 3.2 1B via llama.cpp on the Pi 5. Slower (~500ms inference) but fully offline. Quality is lower than Claude API but better than templates.

## 7.3 TTS (Converting Text to Voice)

| Option | Library | Quality | Latency | Internet Required | Cost | Recommendation |
| --- | --- | --- | --- | --- | --- | --- |
| edge-tts | pip install edge-tts | High (Microsoft neural) | 500ms-1s | Yes | Free | Phase 1 MVP |
| pyttsx3 | pip install pyttsx3 | Medium (OS voices) | < 100ms | No | Free | Offline fallback |
| Coqui TTS (XTTS) | pip install TTS | High (cloneable voice) | 1-2s on Pi 5 | No | Free | Phase 2 offline |
| ElevenLabs API | pip install elevenlabs | Excellent (custom voice) | 300ms-800ms | Yes | $5/mo (30K chars) | Premium product tier |

Recommendation: edge-tts for MVP using voice 'zh-CN-XiaoxiaoNeural' (warm, friendly female) or 'zh-CN-YunxiNeural' (friendly male). Cache pre-generated audio for common scripts to reduce latency to near-zero.

# 8. Expression System (TFT Screen)

The TFT display shows a character face or avatar that reacts to the child's behavior and the Pomodoro state. This is not ML it is a state machine driving a pre-animated sprite set.

## 8.1 Expression States

| Expression | Triggered By | Animation Style |
| --- | --- | --- |
| IDLE / Standby | IDLE state | Slow blinking, calm face, gentle breathing animation |
| ATTENTIVE | FOCUS state, child is focused | Alert eyes, slight smile, minimal movement |
| HAPPY / ENCOURAGING | GOOD_FOCUS event, break start | Wide smile, eyes curve up, slight bounce |
| CONCERNED | DISTRACTED_GAZE or WRITING_IDLE (Stage 1) | Raised eyebrow, slight frown, head tilt |
| FIRM / SERIOUS | PHONE_DETECTED, OUT_OF_SEAT (Stage 3) | Narrowed eyes, flat mouth, slow shake |
| SLEEPY | FATIGUE_DROWSY | Half-closed eyes, drooping head, yawn |
| CELEBRATING | COMPLETE state, milestone reached | Stars/sparkles, jumping, big grin |
| RESTING | BREAK state | Closed eyes, relaxed smile, slow breathing |

## 8.2 Technical Implementation

- Design expressions as 64x64 or 128x128 PNG sprite sheets. 6-10 frames per expression for looping animation.

- Use pygame to drive the TFT display (via fbdev or SPI). At 128x128 resolution, pygame renders comfortably at 15-30fps.

- Alternatively: Use LVGL (Light and Versatile Graphics Library) in C for lower latency and better TFT support, called from Python via cffi.

- Character design: Commission a 2D character designer or use open-source robot/animal face assets. The character should feel friendly, not authoritative.

- The expression state machine is separate from the Pomodoro state machine but subscribes to its events.

# 9. Parent Dashboard

Parents should be able to check on their child's study session without needing to be in the room. The dashboard is read-only parents observe, the bot intervenes.

## 9.1 Data Points Shown

| Data Point | Update Frequency | Display Format |
| --- | --- | --- |
| Current state (Focusing / On Break / Not Started) | Real-time (polling every 10s) | Status badge |
| Session starts time and elapsed focus time | Real-time | HH:MM counter |
| Pomodoro rounds completed today | End of each round | Number + ring chart |
| Distraction events (type, time, duration) | Per-event | Timeline list |
| Focus score per round (0-100) | End of each round | Bar chart |
| Focus trend over past 7 days / 30 days | Daily | Line chart |
| Posture events | Per-event | Count + types |
| Session summary (AI-generated text) | End of session | Short paragraph |

## 9.2 Architecture

- Backend: FastAPI (Python) running on the same Pi 5. Exposes REST endpoints: GET /session/current, GET /events, GET /history/7d, etc.

- Local dashboard: React app bundled and served by the Pi. Parent on same WiFi opens browser to http://[device-ip]:8080.

- Remote access (optional): Reverse proxy via Cloudflare Tunnel (free tier) or ngrok. No port forwarding needed. Parent accesses from anywhere.

- Mobile: WeChat Mini Program consuming the same REST API via the tunnel URL. Alternatively React Native app.

- Push notifications: When a HIGH priority event fires (phone use, prolonged out-of-seat), send push notification to parent via FCM (Firebase Cloud Messaging, free tier) or WeCom notification.

## 9.3 Data Storage Schema

SQLite database on the device. Schema:

- sessions table: id, child_id, start_time, end_time, rounds_completed, focus_score_avg

- events table: id, session_id, event_type, timestamp, duration_seconds, priority, dismissed_at

- focus_snapshots table: id, session_id, minute_mark, focus_score, writing_score, posture_ok

- children table: id, name, age, focus_duration_pref, tts_voice, character_skin

No raw video or images ever stored. Only derived numeric features and event logs.

# 10. Privacy and Data Security

This is non-negotiable for a product that cameras a child in their room. The architecture must be designed for privacy from day one.

| Concern | Risk | Mitigation |
| --- | --- | --- |
| Raw video storage | Footage of minor in private space | NEVER store raw frames. All inference is done in-memory and discarded. No video file is ever written to disk. |
| Cloud data leakage | Session data sent to third parties | All data is local by default. Cloud sync is opt-in, explicit, and uses end-to-end encryption. |
| Unauthorized viewing | Someone accessing parent dashboard without permission | Dashboard requires PIN login. Session tokens expire after 24 hours. |
| Camera indicator | Child unaware they are being observed | Hardware LED hardwired to camera power rail. Physically impossible to run camera without LED being on. LED is prominently visible. |
| Data deletion | Parent wants to erase history | One-tap full data wipe from app. SQLite database file deleted. No cloud backup remains if sync was disabled. |
| Child consent | Child is surveilled without understanding | First-use wizard explains to both parent AND child what the bot does and does not do. Child can see their own focus stats. |
| COPPA / GDPR compliance | Regulatory risk | No data leaves device without explicit parent opt-in. No advertising. No third-party analytics on child data. |

# 11. Critical Weaknesses Honest Assessment

These are the most likely failure modes. Address them in the design before they become user complaints.

## 11.1 False Positive Problem (Highest Risk)

The biggest threat to this product's adoption is not technical failure it is the bot nagging a child who is actually studying. Children stare at the wall while thinking. They pause between sentences. They look at reference books. All of this looks like 'distraction' to a naive detection system.

Why this kills the product: A child who gets incorrectly interrupted 2-3 times per session will hate the bot within a week. They will cover the camera, game the system, or beg parents to disable it.

Mitigations:

- Multi-signal confirmation: NEVER alert on a single signal. Require 2+ signals simultaneously. Example: face off-camera PLUS no writing activity PLUS phone detected = distraction. Face off-camera alone = not sufficient.

- Sustained detection: Any distraction must last at least 20-30 seconds before triggering Stage 1 alert. 60 seconds before Stage 2 voice alert.

- Learning period: First 3 sessions, bot only logs events and does not alert. Uses this data to establish the child's baseline (they naturally look away briefly at this frequency, their writing has this pattern).

- Confidence threshold: Only fire rules if CV confidence > 0.75. Low confidence = skip, do not alert.

- Parent override: Parent can mark events as false positives from the dashboard. System learns over time.

## 11.2 Lighting and Environment Variability

MediaPipe and YOLO accuracy degrade significantly in: low light (< 50 lux), side backlighting, glasses with strong lens glare, very dark skin tones (face mesh sometimes struggles), multiple children in frame.

Mitigations:

- PREPARE phase: Run an automatic environment check. Require: mean frame brightness > 60, face detection confidence > 0.8 for 5 consecutive frames. Do not start session if check fails.

- During session: Monitor rolling detection confidence. If confidence drops below 0.5 for 30+ seconds, pause alerting and display 'camera check' indicator rather than generating false alerts.

- Hardware: Include a small LED ring light around the camera that activates in low-light conditions (auto-detected via ambient light sensor or frame brightness).

## 11.3 Child Resistance and Gaming the System

Children are remarkably creative at circumventing surveillance. Common workarounds: sitting still with blank face while actually daydreaming, placing phone on lap below camera view, having a sibling sit in front of the camera.

Mitigations:

- Position the product as a study companion, not a surveillance device. The bot should feel helpful, not punitive.

- Give the child their own dashboard and ownership over their focus scores. Children respond better to self-improvement framing than parent monitoring framing.

- Gamification: Focus streaks, weekly goals, level-up mechanics. The child should want to engage with the system.

- Do not try to catch every cheat. The goal is habit formation, not perfect enforcement.

## 11.4 Writing Activity Detection Ambiguity

Optical flow cannot distinguish between: writing homework, drawing, doodling, playing with a toy on the desk. It only measures motion in the desk ROI. High motion during focus = 'probably writing'. Low motion = 'possibly idle'.

Mitigations:

- Treat writing activity score as a supporting signal only, not a primary alert trigger. Combine with gaze direction and face presence.

- Phase 2: Fine-tune a small MobileNet classifier on 1-2 hours of labeled video: (writing homework, drawing/doodling, idle, other). Adds specificity.

## 11.5 Single Point of Failure on Edge-TTS

edge-tts requires internet. If the WiFi drops, the primary TTS fails. This is especially bad if the bot then silently stops working and the child does not realize.

Mitigation:

- Pre-generate and cache audio for all ~225 template scripts at startup. If edge-tts connection fails, use cached MP3 files. New LLM-generated scripts fall back to templates.

- pyttsx3 as automatic fallback if cache miss.

## 11.6 Pi 5 Thermal Management

Running MediaPipe + YOLO continuously at 30fps on Pi 5 generates significant heat. Without active cooling, CPU throttling degrades performance.

Mitigation: Include active cooling (official Pi 5 Active Cooler, $5) in the enclosure design. Mandatory for a product meant to run 2–4-hour sessions.

# 12. Development Roadmap

Build in phases. Ship something usable early, validate with real children, then add complexity.

## Phase 1: Software MVP Android/iOS App (Months 1-3)

Goal: Prove the core detection loop works on a real child. Validate false positive rates. No hardware.

- Set up Python pipeline: OpenCV + MediaPipe + YOLOv8n

- Implement feature aggregation layer

- Build Pomodoro state machine (transitions library)

- Implement rules engine with configurable thresholds

- Build template-based script system (225 scripts in Chinese)

- Integrate edge-tts with fallback to pyttsx3

- Build FastAPI parent dashboard backend

- Build basic React dashboard (web, desktop browser)

- Package as Kivy app (cross-platform Python) or wrap in React Native for phone deployment

- Beta test with 5-10 families. Measure: false positive rate per session, session completion rate, child satisfaction.

## Phase 2: Enhanced Detection + LLM (Months 4-5)

Goal: Reduce false positives further. Make reminders feel personal.

- Add per-child baseline calibration (first 3 sessions are learning-only)

- Implement multi-signal confirmation logic in rules engine

- Add posture detection (slouch, too-close-to-screen)

- Add fatigue detection (EAR + head droop + blink rate)

- Integrate Claude API for LLM script generation (with template fallback)

- Add gamification: focus streaks, weekly scores, achievement badges

- Add child-facing dashboard (child sees their own data, not just parents)

- WeChat Mini Program for parent dashboard (CNY market)

## Phase 3: Hardware Desk Bot (Months 6-8)

Goal: Ship the physical product. Hardware must be manufacturable at target cost.

- Finalize compute: Raspberry Pi 5 + Hailo-8L HAT

- PCB design for power management and peripheral integration

- 3D-print enclosure prototypes (target camera angle: 30-45 degrees above eye level)

- TFT display integration with pygame character animation system

- Hardware mic array with wake word detection (custom wake word using openWakeWord library)

- LED ring light with auto-brightness control

- Physical LED camera indicator (hardwired, cannot be software-disabled)

- OS image: custom Raspberry Pi OS with all dependencies pre-installed, auto-start on boot

- Enclosure manufacturing: injection mold quotation or high-quality SLA 3D print for first 50 units

## Phase 4: Scale and Product Polish (Months 9-12)

Goal: Product-market fit, distribution.

- Multi-child household support: face recognition to lock tracking to registered child

- AI tutor feature: between Pomodoro rounds, child can ask bot questions (Claude API)

- Progress reports: weekly PDF summary auto-emailed to parent

- Cloud sync with opt-in: access from any device, long-term trend data

- Localization: Traditional Chinese, English

- Regulatory: Check CE/FCC requirements for hardware product in target market

# 13. Team Requirements

| Role | Phase Needed | Key Skills | Estimated Time |
| --- | --- | --- | --- |
| Python ML Engineer | Phase 1-3 | OpenCV, MediaPipe, YOLO, Python | Full-time Phase 1-2, part-time Phase 3-4 |
| Embedded/Hardware Engineer | Phase 3 | Raspberry Pi, PCB design, Linux, C/Python | Full-time Phase 3 |
| Frontend Developer | Phase 1-4 | React, React Native, WeChat Mini Program | Part-time throughout |
| Character/UI Designer | Phase 2-3 | 2D animation, sprite design, UI/UX | Contract for expression assets |
| Child Development Consultant | Phase 1-2 | Child psychology, age-appropriate UX | Part-time advisory |
| QA / Test Families | Phase 1-4 | Real children aged 6-14 | Ongoing beta testing |

# 14. Cost Summary

## 14.1 Development Costs (One-Time)

| Item | Estimated Cost |
| --- | --- |
| Phase 1 development (Python engineer, 3 months) | $9,000 - $15,000 |
| Phase 2 development (same engineer, 2 months) | $6,000 - $10,000 |
| Phase 3 hardware development (embedded engineer, 3 months) | $9,000 - $18,000 |
| Character design and animation assets | $1,500 - $3,000 |
| Script writing (225 Chinese scripts, native speaker) | $500 - $1,000 |
| Beta testing incentives (10-20 families) | $500 - $1,000 |
| Prototype hardware (10 units) | $2,000 - $3,000 |
| TOTAL ESTIMATED DEVELOPMENT | $28,500 - $51,000 |

## 14.2 Per-Unit COGS (Hardware Bot)

| Item | Unit Cost (USD) | At 500 Units |
| --- | --- | --- |
| Raspberry Pi 5 4GB | $60 | $30,000 |
| Hailo-8L HAT | $50 | $25,000 |
| Camera + display + mic + speaker | $66 | $33,000 |
| PCB + enclosure manufacturing | $25 | $12,500 |
| Assembly + QA | $15 | $7,500 |
| Packaging | $5 | $2,500 |
| TOTAL COGS per unit | ~$221 | ~$110,500 |

Target retail price: CNY 899 - 1299 (~$125-$180). Margin is thin at this price point without volume. App subscription model ($3-5/month) significantly improves unit economics.

# 15. Quick-Start Checklist — What to Do First

If you are starting today, this is the order of work:

- Install dependencies: pip install mediapipe ultralytics opencv-python edge-tts transitions fastapi

- Write a 200-line test script that opens a webcam, runs MediaPipe Face Mesh + YOLOv8n simultaneously, and prints the feature vector to console every second. Verify it runs at 10+ fps on your development machine.

- Build the Pomodoro state machine as a standalone class. Test all 6 state transitions with mock inputs (no camera).

- Build the rules engine as a standalone class. Feed it simulated feature vectors and verify it generates the correct events.

- Wire TTS: pass a string to edge-tts and play the output audio. Verify the full round-trip works.

- Connect everything: camera → features → rules → TTS. Run a real 25-minute study session on yourself. Count false positives.

- Build the FastAPI backend and React dashboard. Display a live session in the browser.

- Find 3 children aged 7-12. Run real sessions. Observe. Adjust thresholds. This is where 80% of the real learning happens.

The technology is all proven and available. The hard part is calibration, UX, and getting children to like it. Start there, not with the hardware.